Synchronous Buck Converter Circuit Component Selection Equations:

- Mode- Continuous Conduction Mode

Input Parameters:

VIN MIN(Min Output Voltage) in Volts

VIN MAX (Max Input Voltage) in Volts

VOUT MIN (Min Output Voltage) in Volts

VOUT MAX (Max Output Voltage) in Volts

POUT MAX (Max Output Power) in Watts

Eff from 0 to 1

Fs (Switching Frequency) in Hz

VRIPPLE MAX (Output Voltage Ripple) in Volts

VIN RIPPLE (Input Voltage Ripple) in Volts

IOUT RIPPLE (Inductor Current Ripple) Range ( Min, Max) in Amps

Load Transient Requirements:

VOVERSHOOT (Overshoot value (in volts)

VUNDERSHOOT Undershoot Value in volts.

ILOADSTEP Load Step (in Amps)

INDUCTOR:

Inductor L(in Henry)=(VIN MAX-VOUT MIN)×DMAXFs×IOUT RIPPLE

DMAX=VOUT MAXVIN MIN

- While selecting inductor, Saturation current > Peak inductor current,

- Low DCR.

- Low cost

OUTPUT CAPACITOR:

COUT MIN=IOUT RIPPLE8×Fs ×VRIPPLE MAX  (Considering Vout ripple )

COUT MIN=L×ILOADSTEP22 ×VUNDERSHOOT×(VIN MAX-VOUT MIN)×DMAX  (Considering Undershoot)

COUT MIN=L×ILOADSTEP22 ×VOVERSHOOT×VOUT MAX (Considering Overshoot)

- Among Ceramics and Electrolytic capacitors, ceramics are preferred since they have low ESR values. Once selected the Capacitor, parse the datasheet for following information Cost, ESR, ESL Voltage coefficients, leakage currents, lifetime.

INPUT CAPACITOR:

CIN=IIN×DMAXFs ×VIN RIPPLE  (Considering Vin ripple )

IIN=POUT MAXEff×VIN MIN

SI MOSFETS/ SIC MOSFETS:

For both HIGH SIDE MOSFETS and LOW SIDE MOSFETS:

- Select SI MoSFETs or SIC MOSFETS based on voltage rating greater than Max Output Voltage and having lowest RDSon resistance (take from Datasheet), consider cost and reliability as well.

Notes: In case of capacitor, if load transient requirements are provided, then select based upon  load transient as well as output voltage ripple. If not provided, then consider Vout ripple only. But do consider ESR as well because that affects Control loop stability as well (because it creates a zero after the double pole frequency). Load transients are essential for power supplies supply sensitive loads such as MCU and similar loads.