Totem Pole PFC Circuit Component Selection Equations:

Inductor L( in Henry)=VOUT MAX12×Fs×IRIPPLE MAX

IRIPPLE MAX=0.1×2×POUT MAXVIN MAX×Eff

COUT MIN=POUT MAX4×π×fLine_min×VRIPPLE MAX×VOUT

Select SIC MoSFETs based on voltage rating greater than Max Output Voltage and having lowest RDSon resistance( take from Datasheet), consider cost and reliability as well.

Input Parameters:

VIN MIN(Min Output Voltage) in Volts

VIN MAX (Max Input Voltage) in Volts

VOUT MIN (Min Output Voltage) in Volts

VOUT MAX (Max Output Voltage) in Volts

POUT MAX (Max Output Power) in Watts

Eff from 0 to 1

Fs (Switching Frequency) in Hz

fLine_min (Input AC line Frequency) in Hz

VRIPPLE MAX (Output Voltage Ripple) in Volts

Output :

Inductance L in Henry

Capacitance in Farad

MoSFET PART number with its Rds ON